A unit test checks one small piece of the system on its own, with an input where we already know the right answer. If someone changes the code later and breaks that piece, the test fails straight away. That's why I rerun them after every change.

The 13 tests are all in mg_hier/tests/test_hier.m. Run them in MATLAB with runtests('tests/test_hier.m').

| # | Test | What it checks | Why it matters |
| --- | --- | --- | --- |
| 1 | Power flow, single line | One house on one cable: the computed voltage matches the exact textbook formula | The physics engine calculates correctly |
| 2 | Reactive power drops voltage | More reactive power gives lower voltage, and exporting solar raises it | The model behaves like a real grid |
| 3 | Node critical hold | Once switched on, a "switch critical off" command is ignored | Critical loads can never be cut by a command |
| 4 | Node comms loss | After 5 s with no command, flexible goes OFF and critical stays ON; flexible comes back when the link returns | The safe fallback works |
| 5 | Node over-current lockout | Over 40 A, the node trips, retries after 10 s, and locks out on the second trip | Local protection works |
| 6 | Node energy meter | 3,600 W for 10 s adds exactly 10 Wh, and export is counted separately | Billing is right |
| 7 | Battery hard limits | No discharge at or below 30% charge, no charging at or above 95%, and the inverter rating is respected; it holds the last setting if the link is lost | The battery is protected whatever it's told |
| 8 | Main-controller input check | The main controller refuses any input other than node reports, the grid-connection meter and Pi requests | It can't "cheat" by reading the simulation's hidden values (the "what's real?" answer) |
| 9 | Main controller never cuts critical | Even a "cut critical" request leaves critical loads on, and the request is refused | Same rule, enforced at the top layer too |
| 10 | Message round trip | Every text line between MATLAB and the ESPs reads back exactly as written | MATLAB and the boards see identical numbers |
| 11 | MQTT access rules | H1 can't read H2's data or act as the operator; anonymous users get nothing; the AI can't fake measurements | Dashboard privacy and security |
| 12 | Add a house | A third house works without changing any code | The design scales |
| 13 | Live link to the Pi (stand-in broker) | A JSON request is accepted, garbage is refused without crashing, one state message is sent per tick | The MATLAB-to-Pi connection works |

The unit tests are one of three layers of checks:

- Unit tests (13) check one piece at a time.

- Scenarios (13) run the whole microgrid through a story, such as an outage, low battery or a faulty house, and check the rules for each.

- Monte Carlo (40 runs) uses random situations with random faults, to catch cases nobody thought of.

On top of those, the software-in-the-loop test (SIL) compares the real ESP32 code with MATLAB, reply for reply.