PS #38 Microgrid: Roadmap after the hackathon (proposed 2026-10-09)

Status: PROPOSAL for the team to agree. Goal: learning and a career in sustainable energy, ending in a credible, data-backed microgrid model of our own campus.

Priority order (merged and re-ordered from the team's 11 ideas)

| Rank | Item | Why this position | Effort |
| --- | --- | --- | --- |
| 0 | Debrief: ask judges for feedback; tag the code; README + 3-min demo video | Cheap; tells us what was missing; the video is the base for sharing later | S |
| 1 | (4) Real datasets | Everything later is only as believable as the input data. Solar: NASA POWER / PVGIS for Sangli. Loads: Prayas EMARC smart-meter data (check access terms); later, campus meter data | S-M |
| 2 | (6) Ledger with ToD, net metering, real tariffs | Highest learning per hour; energy economics is a job skill. Use the current MERC tariff order (ToD slots, rates), MSEDCL net-metering rules, compare prepaid P2P vs net metering vs ToD, in Rs per house per month | M |
| 3 | (5)+(2) Validate in ETAP or OpenDSS, incl. harmonics | Cross-check our power flow; add short-circuit, protection and harmonic studies our 1 s phasor model cannot do. Harmonics belong in a harmonic load-flow tool (check THD against IEEE 519), not in our MATLAB model. ETAP if the department has a licence, else OpenDSS (free, EPRI) | M-L |
| 4 | (7) Validation the way industry does it | Not just more Monte Carlo: benchmark against a reference (item 3), sensitivity analysis, checks against standards (IEEE 1547 / CEA connectivity rules: voltage, frequency, islanding times), Monte Carlo on real data | M |
| 5 | (11) Campus microgrid model (north star) | Real data, real stakeholders, real decisions (DG fuel, solar, battery, ToD savings). It absorbs (3): add real components (DG, labs, rooftop PV, EV) instead of imaginary ones. Use HOMER Pro for techno-economic sizing if a licence is available | L |
| P | (1) Dashboard control + forecasting | AI/ML team, in parallel. The plumbing exists (mg/cmd topics, main-controller checks, ACL, DASH_INPUTS flag). Forecasts become meaningful once item 1 gives real data | M (their team) |
| P | (8) Communication protocol | Hardware side-track. Learn Modbus RTU over RS-485 first (real meters and inverters speak it; PZEM-004T uses it). CAN (ESP32 has a built-in TWAI controller + SN65HVD230 transceiver) for a deterministic node bus. Keep MQTT for Pi/dashboard. IEC 61850: awareness only | M |
| Later | (10) Industry sharing | Light version now (video, GitHub, LinkedIn post). Full version after items 3-4 give validated numbers | S |
| Later | (9) NETRA (NTPC R&D), MSEDCL | Go through a faculty mentor with a specific ask: MSEDCL feeder/tariff data or a substation visit; NETRA visit or internship. Formal collaboration is unlikely without a faculty PI | S per contact |

Phases

- Phase 0 (this week): debrief, freeze, document, demo video.

- Phase 1: datasets + ledger/ToD.

- Phase 2: ETAP/OpenDSS validation incl. harmonics + standards checks + Monte Carlo on real data.

- Phase 3: campus model (DG, labs, solar, grid), techno-economics, report/paper.

- Parallel all along: AI/ML dashboard control and forecasting; protocol side-track.

Career value of the campus model (honest view)

Skills it builds and where they are used:

- Power system studies (load flow, short circuit, harmonics, protection) in ETAP/OpenDSS: EPC firms, consultancies, utilities.

- Solar + BESS + DG sizing and economics (HOMER/PVsyst-type work): solar developers, C&I energy firms.

- Energy auditing and tariff/ToD analysis: energy service companies, campus and industrial energy managers (BEE energy auditor exam later).

- Microgrid controls / EMS / SCADA (what we already built): automation and grid-tech companies.

- Research path: M.Tech/MS in microgrids or power electronics. What makes it count: real data, real stakeholders, a number at the end (Rs saved, kWh, tCO2, DG hours avoided). Depth and a finished result beat a long feature list. Caveat: it depends on getting campus data (bills, meter logs, DG logs) and a faculty mentor; start those requests early.

Open checks

- Does the department have ETAP / PSCAD / HOMER licences?

- Can the estate/electrical maintenance office share 1 year of bills, meter data and DG logs?

- Prayas EMARC data access terms.