Yes. For your team, I would not ask the hardware person to build a huge new sensing system just for the dashboard. The goal should be:

Make the hardware expose clean, timestamped energy data and basic control/status signals. Let the dashboard + AI do the intelligence.

Your brief is essentially built around three houses, rooftop PV, a shared BESS, grid connection/islanding, household loads and household energy credits.

## 1. The minimum data the hardware must give us

This is the actual hardware → dashboard/AI data contract.

| Data | H1 | H2 | H3 | BESS | Grid | Why we need it |
| --- | --- | --- | --- | --- | --- | --- |
| Voltage | ✅ | ✅ | ✅ | ✅* | ✅ | System health / validation |
| Current | ✅ | ✅ | ✅ | ✅ | ✅ | Calculate power if needed |
| Active Power (W) | ✅ | ✅ | ✅ | ✅ | ✅ | Most important live dashboard value |
| Energy (Wh/kWh) | ✅ | ✅ | ✅ | ✅ | ✅ | Billing, savings, P2P credits |
| Import / Export | ✅ | ✅ | ✅ | — | ✅ | Know where energy is flowing |
| PV generation | ✅ | ✅ | ✅ | — | — | Solar production/forecast |
| Load consumption | ✅ | ✅ | ✅ | — | — | Demand forecast |
| Battery SOC | — | — | — | ✅ | — | Core optimization input |
| Battery charge/discharge power | — | — | — | ✅ | — | Dispatch + live flow |
| Battery voltage/current | — | — | — | ✅ | — | BESS health/control |
| Battery temperature | — | — | — | 🟡 | — | Safety / diagnostics |
| BMS fault/status | — | — | — | ✅ | — | Safe AI control |
| Grid available? | — | — | — | — | ✅ | Outage/islanding logic |
| Grid voltage/frequency | — | — | — | — | ✅ | Detect abnormal grid |
| Breaker/contact status | — | — | — | — | ✅ | Confirm islanding |
| Critical load ON/OFF | ✅ | ✅ | ✅ | — | — | Resilience metric |
| Flexible load ON/OFF | ✅ | ✅ | ✅ | — | — | Load shedding |
| Relay/contactor feedback | ✅ | ✅ | ✅ | ✅ | ✅ | Verify commands actually happened |

* depending on how your BESS/inverter exposes data.

### The six absolutely critical inputs

If your hardware team says, "Give us the minimum list; we have limited time," give them this:

Per household

- PV power

- Load power

- Net import/export power

- Cumulative imported/exported energy

- Critical/flexible load status

- Household meter ID + timestamp

Shared BESS
7. SOC
8. Charge/discharge power
9. BMS/inverter status/fault

Grid
10. Grid ON/OFF
11. Grid import/export power
12. Voltage/frequency

That is enough to make a surprisingly strong first version.

# 2. VERY IMPORTANT: P2P credits need house-level metering

This is the one thing I'd specifically tell your hardware guy.

You said:

rooftop → grid is already working
energy sharing is already setup
but credit between users isn't available yet

Then the missing ingredient isn't another fancy AI model. It's trustworthy household-level energy accounting.

For example:

HOUSE 1

PV = 800 W

Local Load = 300 W

Surplus = 500 W

│

├── 300 W → H1

│

└── 500 W → Community bus

│

├── 350 W → H3

└── 150 W → BESS

The software can then record:

H1 supplied H3

Energy = 0.350 kWh

Rate = ₹6.20/kWh

Credit to H1 = ₹2.17

Debit to H3 = ₹2.17

But the dashboard cannot legitimately calculate that just from one community meter.

We need to know:

Who generated it? Who consumed it? How much? When?

So each house needs its own energy measurement point.

### Ideally, each house should expose

House ID

Timestamp

PV Power

Load Power

Net Power

Import Energy

Export Energy

Then your software can calculate the P2P ledger.

# 3. What the physical hardware setup should look like

I would ask the hardware team to structure it like this:

┌────────────────────┐

│   CENTRAL GATEWAY   │

│ Raspberry Pi / PC   │

│ MQTT + Database     │

└─────────┬──────────┘

│

┌──────────────────┼──────────────────┐

│                  │                  │

▼                  ▼                  ▼

┌─────────┐        ┌─────────┐        ┌─────────┐

│ HOUSE 1 │        │ HOUSE 2 │        │ HOUSE 3 │

│ Energy  │        │ Energy  │        │ Energy  │

│ Meter   │        │ Meter   │        │ Meter   │

└────┬────┘        └────┬────┘        └────┬────┘

│                  │                  │

PV / Load          PV / Load          PV / Load

│                  │                  │

└──────────────────┼──────────────────┘

│

▼

COMMUNITY BUS

│

┌────────┴────────┐

│                 │

▼                 ▼

🔋 BESS          ⚡ GRID

│                 │

SOC / P / V/I      status / P / V / f

And separately:

House 1 ──┐

House 2 ──┼──→ controllable loads / relays

House 3 ──┘

│

▼

Critical / Flexible

# 4. What hardware actually needs to install

## A. Each house

### Energy meter

Use an energy meter that can provide at least:

V, I, W, Wh, import/export

For the prototype, this can be a suitable AC smart meter/energy-meter module with a digital interface such as RS-485/Modbus, depending on what your existing setup already uses.

You don't necessarily need a separate current sensor for every quantity if the meter already exposes these measurements.

### Ideally measure at two points

For each house:

PV output ──→ [PV meter]

│

▼

House bus

│

[Load meter]

│

▼

Loads

This gives you real:

PV generation ≠ house consumption

rather than trying to infer everything from one measurement.

# 5. Shared BESS hardware interface

Your AI/MPC cannot intelligently control the battery from SOC alone.

At minimum:

SOC              72 %

Battery Power    +18 W

Battery Voltage  24.6 V

Battery Current  0.73 A

State            CHARGING

BMS Status       NORMAL

Max Charge       20 W

Max Discharge    30 W

The Max Charge / Max Discharge values are particularly useful because the optimizer should not simply decide:

"Charge battery at 50 W"

when the hardware can only safely accept 20 W.

So ideally the inverter/BMS should expose:

- SOC

- battery power

- voltage

- current

- temperature

- charge limit

- discharge limit

- fault/status

- command acknowledgement

# 6. Grid hardware interface

You need much more than a green "Grid Connected" icon.

The dashboard should know:

Grid status       CONNECTED

Voltage           230.4 V

Frequency         49.96 Hz

Import power      42 W

Export power      0 W

Breaker           CLOSED

During the outage demo:

Grid status       FAILED

Breaker           OPEN

Island mode       ACTIVE

Battery           DISCHARGING

That makes the resilience portion visually convincing.

Your brief explicitly treats grid availability and islanding as inputs/scenarios for the manager.

# 7. Controllable loads

You don't need to instrument every appliance individually for the hackathon.

For the dashboard, classify loads as:

HOUSE 1

Critical

Refrigerator

Lights

Flexible

Water heater

Pump

EV charger

Hardware needs to give:

Load ID

ON/OFF

Actual power

Relay/contactor state

And receive:

TURN ON

TURN OFF

with confirmation:

Command: OFF

Feedback: OFF ✓

That last part is important.

Otherwise your dashboard might say:

AI shed flexible load

while the physical load is actually still running.

# 8. Communication setup

This is the part I'd standardize before the hardware team builds anything.

Every reading should look conceptually like:

{

"house_id": "H1",

"timestamp": "2026-10-06T01:45:12",

"pv_power_w": 38.4,

"load_power_w": 21.7,

"net_power_w": 16.7,

"import_wh": 2314.2,

"export_wh": 884.6

}

BESS:

{

"soc": 72.4,

"power_w": -18.2,

"voltage_v": 24.6,

"current_a": -0.74,

"status": "CHARGING",

"fault": false

}

Grid:

{

"available": true,

"voltage_v": 230.4,

"frequency_hz": 49.96,

"power_w": 42.1

}

### Recommended architecture

Sensors / Meters

↓

ESP32 / PLC / Meter Gateway

↓

MQTT / Modbus

↓

Central Pi / Server

↓

Time-series database

↓

AI + MPC

↓

Dashboard

↓

Commands back to hardware

This is much better than making the dashboard directly talk independently to ten different sensors.

# 9. What the hardware does NOT need to handle

This is important because otherwise your hardware guy may start implementing software logic.

### Don't put these in hardware:

AI forecasting

The hardware only sends measurements.

MPC optimization

Software.

P2P price calculation

Software.

Household credits

Software database/ledger.

"Why did AI choose this?"

Software.

Cost/savings calculation

Software.

24-hour solar forecast

Software/external weather data.

AI vs rule-based comparison

Software.

So the hardware team's job is basically:

Measure accurately. Report reliably. Execute commands. Confirm execution.

# 10. What I would prioritize for your hackathon

Since you said your dashboard is going to be the standout feature, I'd divide the work like this:

### MUST HAVE

H1 PV + load + energy

H2 PV + load + energy

H3 PV + load + energy

BESS SOC + power + status

Grid status + power

Critical/flexible load state

Reliable timestamps

Two-way command + acknowledgement

### NICE TO HAVE

Battery temperature

Grid frequency

Individual load power

Weather/irradiance sensor

Detailed BMS parameters

Power factor

Voltage quality

### DON'T WASTE TIME ON YET

Per-appliance NILM

Extra environmental sensors

Fancy hardware displays

Too many redundant sensors

The judges don't care that you measured 17 different parameters if the dashboard cannot turn them into a visible decision.

# 11. The dashboard should then be able to show this

With just the above hardware data, your frontend can produce:

╔══════════════════════════════════════════════════════╗

║ COMMUNITY ENERGY MANAGER                             ║

║                                                      ║

║  SELF SUFFICIENCY     BESS       GRID       SAVING  ║

║       78%             72%        ONLINE       ₹84    ║

╚══════════════════════════════════════════════════════╝

LIVE ENERGY FLOW

☀ H1                 ☀ H2

38W                  25W

│                    │

└───────┬────────────┘

▼

┌────────────────┐

│ COMMUNITY BUS  │

└───────┬────────┘

↙  ↓  ↘

H1   H3  🔋

22W 18W

🧠 AI DECISION

"Charge BESS at 18 W"

Reason:

• Solar surplus available

• Evening demand predicted ↑ 14%

• Grid price expected to increase

• Maintain 65% outage reserve

P2P ENERGY MARKET

H1 → H3     1.4 kWh

₹6.20/kWh   → ₹8.68

OUTAGE RESILIENCE

Critical loads served     3/3 ✓

Estimated backup time     4h 12m

Flexible load shed        0.7 kWh

That entire screen is possible from the hardware inputs above.

# The one thing I would tell your hardware guy

I would send them this exactly:

## Hardware → Dashboard Requirements

Our dashboard/AI layer needs the hardware to expose reliable, timestamped measurements and basic actuator feedback. Please prioritize data availability over adding extra sensors.

### 1. House-level measurements — H1, H2, H3

For EACH house, provide:

- PV generation power (W)

- House load power/consumption (W)

- Net power flow (W)

- Imported energy (Wh/kWh)

- Exported energy (Wh/kWh)

- Voltage

- Current

- Timestamp

- Unique house ID

Preferably, PV and household load should be measured separately. This is important because we need to know how much energy belongs to each household for P2P credit calculation.

### 2. Shared BESS

Provide:

- SOC (%)

- Battery charge/discharge power (W)

- Battery voltage

- Battery current

- BMS/inverter operating status

- Fault/alarm status

- Maximum allowed charge power

- Maximum allowed discharge power

- Battery temperature if available

The AI optimizer must know the battery's actual limits before sending charge/discharge commands.

### 3. Grid

Provide:

- Grid available / unavailable

- Grid import power

- Grid export power

- Grid voltage

- Grid frequency

- Breaker/contactor status

- Islanded/grid-connected status

A direct grid availability/breaker signal is preferred so that outage detection does not rely only on a sudden power drop.

### 4. Controllable loads

For critical and flexible loads, provide:

- Load ID

- ON/OFF state

- Actual power if possible

- Relay/contactor state

- Command acknowledgement

The software will decide which flexible loads to shed. Hardware only needs to execute and confirm the command.

### 5. Communication

All measurements should reach the central gateway through a common interface such as:

Meter/Sensors → ESP32/PLC/Gateway → MQTT/Modbus → Central Computer → Database → AI/MPC → Dashboard

Each message should contain:

- timestamp

- device/house ID

- measurement

- unit

- status/fault/quality flag

We want reliable periodic telemetry and a return path for control commands.

### 6. P2P credit requirement — VERY IMPORTANT

The existing rooftop/grid energy flow is not enough by itself for our new community-credit feature.

We need trustworthy house-level measurements so that the software can determine:

H1 generated/supplied → X Wh
H2 consumed → Y Wh
H3 bought → Z Wh

The dashboard/software will then calculate:

- who supplied energy

- who consumed it

- amount traded

- P2P price

- buyer debit

- seller credit

- community savings

Do NOT implement the credit logic in hardware.

### 7. Priority

MUST HAVE:

H1/H2/H3 PV + load + import/export energy
BESS SOC + charge/discharge power + status
Grid availability + import/export power
Critical/flexible load status
Timestamped communication
Command + acknowledgement

NICE TO HAVE:

Battery temperature
Power factor
Detailed BMS parameters
Individual load power
Additional power-quality measurements

The main differentiator of this project will be the dashboard and AI decision layer, so hardware should focus on giving us clean and trustworthy data rather than overcomplicating the sensing system.

### And one strategic point for your team

I think your instinct is right: don't try to win by inventing another energy-sharing circuit.

The energy-sharing mechanism can be the infrastructure.

Your dashboard becomes the brain and the story:

Where is energy going? → What will happen next? → What did AI decide? → Why? → Who benefited? → How much money/energy did we save? → What happens during an outage?

The one hardware addition I would fight hardest for is proper per-house bidirectional metering. Without that, your P2P credit feature risks being a simulated number on the dashboard; with it, the dashboard can show a genuinely traceable transaction.